STUDENT NUMBER: 23018192

MODULE CODE: COMP1421

ASSIGNMENT NUMBER: 3

DUE DATE: 14/03/2024

COMP1421 - Assignment 3

Foundation to Computing

Report with individual presentation

Table of Content

Introduction	3

Focus Point - Obstacle Avoidance	3

Obstacle Avoidance – Arduino Code with annotations	3

Results	8

Conclusion	8

# Introduction

This project is based on the movements and the understanding of how various components of a parallax robot that contains an Arduino board.

# Focus Point - Obstacle Avoidance

Obstacle avoidance is the ability for an object (in this case a robot) to detect (using sensors) that there is an obstacle ahead of it that may disrupt it course or route.

Examples of this can be found in delivery robots developed by Starship Technologies that allows users to be able to order their groceries/food from a store that offers this delivery option. These robots are responsible for delivering the groceries/food to the customers and have sensors that will allow it to be able to avoid obstacles like people but also identify incoming cars to be able to reach the destination safely.

# Obstacle Avoidance – Arduino Code with annotations

<studio.h> and <Servo.h> are both under the brackets of libraries/components. In specific the ones mentioned above define pins and constants for the setup.

HC_SR04_Tx & HC_SR04_Rx define the ultrasonic sensors.

ServoL & ServoR define the pin number for the various components used in the project, such as the sensors & the left and right servo motors.

- DEBUG is used for enabling/disabling outputs.

- wheelDiam is to define the diameter of the wheels.

- axleLen is used to define the length of the axle.

#include <stdio.h>

#include <Servo.h>

// Define pins ---------------

#define HC_SR04_Tx 2

#define HC_SR04_Rx 3

#define ServoL_pin 13

#define ServoR_pin 12

// Define constants ----------

#define DEBUG true

#define wheelDiam 66.0

#define wheelRad (wheelDiam/2.0)

#define axleLen 104.0

There are 3 main finite state machines (FSM) states that we set the robot to perform the object avoidance:

forward – which makes the robot move forwards, backwards – which make the object move backwards and turn right – which makes the robot turn to the right by the amount that the set.

This is used when the sensors of the robot identify and obstacle while moving forwards, as soon as identified it will reverse, moving backwards and will turn to the left to avoid the obstacle.

ServoL & ServoR used to control the left and right servo.

DriveL & DriveR are used to store the values of ServoL and ServoR.

Dist holds the distance that the ultrasonic sensor measures.

State indicates the current FSM states (explained above).

The setup() function initialises the serial communication, sets the pin modes for the ultrasonic sensor, attaches servo objects to their respective pins, stops servo movement, adds an initial delay, and sets the initial state to STATE_FORWARD.

// Define states

#define STATE_FORWARD 1

#define STATE_BACKWARD 2

#define STATE_TURN_LEFT 3

// Global variables ----------

Servo servoL;

Servo servoR;

float driveL, driveR;

float dist;

int state;

void setup() {

Serial.begin(9600);

cprintf(">>> Initializing stuff ..................\n");

pinMode(HC_SR04_Tx,OUTPUT);

pinMode(HC_SR04_Rx,INPUT);

servoL.attach(ServoL_pin);

servoR.attach(ServoR_pin);

// Stop any servo movement

driveServos(0.0,0.0);

cprintf(">>> Pausing 2 secs ......................\n");

delay(2000);

cprintf(">>> Starting  ...........................\n");

state = STATE_FORWARD;

}

void loop() {

switch(state) {

case STATE_FORWARD :

// insert code to set driveL and driveR

// and send to servos

driveL=40;

driveR=50;

driveServos(driveL,driveR);

// insert code to get the dist measurement

int delayVal;

// delay of 60 ms required by datasheet

delayVal = 60;

dist = getDistance();

cprintf("dist = %f\n",dist);

delay(delayVal);

The loop()function contains a switch-case statement that handles the states of the FSM.

Depending on the current state, the code block will be executed.

The switch statement evaluates the value of the state variable and executes the code depending on the state. This allows the program based on the current state of the finite state machine (FSM).

// test the dist and transit to next state

if ( (dist < 150) && (dist != -1) ) {

cprintf("Transit to state STATE_BACKWARD\n");

state = STATE_BACKWARD;

}

break;

case STATE_BACKWARD :

// insert code to set driveL and driveR

// and send to servos

driveL=-50;

driveR=-50;

driveServos(driveL,driveR);

// insert a delay for a number of millisecs

delay(1000);

cprintf("Transit to state STATE_TURN_LEFT\n");

state = STATE_TURN_LEFT;

break;

case STATE_TURN_LEFT :

// insert code to set driveL and driveR

// and send to servos

driveL=0;

driveR=50;

driveServos(driveL,driveR);

// insert a delay for a number of millisecs

Each case statement represents a different state of the FSM. Depending on the value of state, the program will execute the code block associated with that specific state.

delay(1200);

cprintf("Transit to state STATE_FORWARD\n");

state = STATE_FORWARD;

break;

}

}

float getDistance() {

long duration;

float mm;

digitalWrite(HC_SR04_Tx, LOW);

delayMicroseconds(2);

digitalWrite(HC_SR04_Tx, HIGH);

delayMicroseconds(10);

digitalWrite(HC_SR04_Tx, LOW);

duration = pulseIn(HC_SR04_Rx, HIGH);

mm = 10*duration / 29 / 2;

// Catch 'bad' values and report as mm = -1;

if(duration < 600) mm = -1;

if(mm > 4000) mm = -1;

//if(DEBUG) cprintf("Dist (mm) = %f\n",mm);

return mm;

}

void driveServos(float _driveL, float _driveR) {

These lines declare two local variables within the function: duration to store the duration of the echo signal and mm to store the calculated distance in millimeters.

This block of code triggers the ultrasonic sensor to send out a pulse. It starts by setting the transmit pin (HC_SR04_Tx) to low, waits for 2 microseconds, sets it to high for 10 microseconds to trigger the sensor, and then sets it back to low.

This line uses the pulseIn() function to measure the duration of the echo signal received by the ultrasonic sensor. It waits for the echo signal to go from LOW to HIGH on the receive pin (HC_SR04_Rx), and then measures the duration of the pulse in microseconds.

These functions are helpers:

- getDistance() measures the distance using the ultrasonic sensor.

- driveServos() controls the servo motors based on the drive values provided.

The remaining code within the loop() function implements each state of the FSM, including setting servo drive values, getting distance measurements, and transitioning between states based on certain conditions.

servoL.writeMicroseconds(1500 + (int)_driveL);

servoR.writeMicroseconds(1500 - (int)_driveR);

}

# Results

After multiple tests to see what the best speed (driveL is 40 and driveR is 50) for the movement of the robot compared to give a good distance (when the distance is less than 150mm and not equals to -1) the most adequate variables to adapt are the ones in the code above, giving us the following result on the YouTube link down below:

https://youtube.com/shorts/d_0r2C55wBk?feature=share

# Conclusion

In conclusion, the project uses a finite state machine (FSM) to control the Arduino robot. The FSM controls the robot's behaviour, allowing it to autonomously navigate its environment by moving forward, backward, and turning left based on the distance measured by the sensor. Ultimately, this project demonstrates the application of FSMs and sensor-based control in robotics.